# Experiment No. 1

## Title

Case Study on Comparison of OSI and TCP/IP Reference Models with Respect to Layers, Functions, Protocols, and Real-World Implementation (Compulsory)

## Date of Completion

# Objectives

- To study the OSI and TCP/IP reference models.

- To compare the layers, functions, and protocols of both models.

- To understand how data travels through different network layers.

- To analyze the practical implementation of the TCP/IP model in modern computer networks.

- To identify similarities and differences between OSI and TCP/IP models.

# Problem Statement

Study and compare the OSI and TCP/IP reference models with respect to their architecture, layers, functions, protocols, advantages, disadvantages, and practical implementation in computer networks.

# Software Requirements

- Cisco Packet Tracer (Optional)

- Wireshark (Optional)

- Microsoft Word / PDF Reader

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

- Internet Connection (Optional)

# Theory

## Introduction

A reference model provides a standard framework for communication between network devices. The two most widely used networking models are:

- Open Systems Interconnection (OSI) Model

- Transmission Control Protocol/Internet Protocol (TCP/IP) Model

The OSI model is a conceptual model developed by the International Organization for Standardization (ISO), whereas the TCP/IP model is a practical protocol suite used on the Internet.

# OSI Reference Model

The OSI model consists of seven layers.

| Layer | Function | Examples |
| --- | --- | --- |
| Application | Provides network services to users | HTTP, FTP, SMTP |
| Presentation | Data encryption, compression, translation | SSL/TLS, JPEG |
| Session | Establishes and manages sessions | RPC, NetBIOS |
| Transport | Reliable data transfer | TCP, UDP |
| Network | Routing and logical addressing | IP, ICMP |
| Data Link | Framing, MAC addressing | Ethernet, PPP |
| Physical | Transmission of bits | UTP Cable, Fiber Optics |

# TCP/IP Reference Model

The TCP/IP model consists of four layers.

| Layer | Function | Protocols |
| --- | --- | --- |
| Application | Combines Application, Presentation and Session layers | HTTP, FTP, DNS, SMTP |
| Transport | End-to-end communication | TCP, UDP |
| Internet | Logical addressing and routing | IP, ICMP, ARP |
| Network Access | Data transmission over physical media | Ethernet, Wi-Fi |

# Comparison between OSI and TCP/IP Models

| Feature | OSI Model | TCP/IP Model |
| --- | --- | --- |
| Developed By | ISO | DARPA |
| Number of Layers | 7 | 4 |
| Nature | Conceptual Model | Protocol Suite |
| Network Layer | Supports Connection-oriented and Connectionless | Connectionless |
| Transport Layer | Connection-oriented and Connectionless | TCP and UDP |
| Practical Use | Mainly Educational | Widely Used on Internet |
| Protocol Dependency | Independent | Protocol Dependent |
| Flexibility | Less Flexible | Highly Flexible |

# Layer Mapping

| OSI Layer | TCP/IP Layer |
| --- | --- |
| Application | Application |
| Presentation | Application |
| Session | Application |
| Transport | Transport |
| Network | Internet |
| Data Link | Network Access |
| Physical | Network Access |

# Common Protocols

| Protocol | Layer |
| --- | --- |
| HTTP | Application |
| HTTPS | Application |
| FTP | Application |
| SMTP | Application |
| DNS | Application |
| TCP | Transport |
| UDP | Transport |
| IP | Internet |
| ICMP | Internet |
| Ethernet | Network Access |

# Real-World Implementation

### Example: Opening a Website (www.google.com)

- User enters URL in browser.

- DNS converts domain name into IP address.

- TCP establishes a reliable connection.

- IP routes packets across the Internet.

- Ethernet/Wi-Fi transmits frames through the network.

- Server sends the requested webpage.

- Browser displays the webpage.

This communication follows the TCP/IP model, although its functions can also be explained using the OSI model.

# Advantages of OSI Model

- Standardized architecture

- Easy troubleshooting

- Layer independence

- Better learning model

# Disadvantages of OSI Model

- Complex architecture

- Rarely implemented completely

- More theoretical than practical

# Advantages of TCP/IP Model

- Practical implementation

- Highly scalable

- Reliable communication

- Internet standard

- Supports heterogeneous networks

# Disadvantages of TCP/IP Model

- Does not clearly separate Session and Presentation layers

- Less modular compared to OSI

- Difficult to replace protocols

# Algorithm

- Study the architecture of the OSI model.

- Study the architecture of the TCP/IP model.

- Compare the number of layers.

- Compare the functions of each layer.

- Identify protocols associated with each layer.

- Analyze the mapping between both models.

- Study the real-world implementation using web browsing.

- Record observations and conclude the comparison.

# Flowchart

Start
             |
             V
Study OSI Reference Model
             |
             V
Study TCP/IP Model
             |
             V
Compare Layers
             |
             V
Compare Functions
             |
             V
Compare Protocols
             |
             V
Analyze Real-world Example
             |
             V
Record Observations
             |
             V
          Conclusion
             |
             V
            Stop

# Test Cases

| Test Case | Expected Result |
| --- | --- |
| Compare number of layers | OSI = 7, TCP/IP = 4 |
| Compare Application Layer | TCP/IP combines three OSI layers |
| Compare Transport Layer | TCP and UDP present in TCP/IP |
| Compare Network Layer | IP used for routing |
| Compare Practical Usage | TCP/IP is used on the Internet |

# Observation Table

| Parameter | OSI | TCP/IP |
| --- | --- | --- |
| Layers | 7 | 4 |
| Developed By | ISO | DARPA |
| Practical Use | Educational | Internet |
| Standard | Reference Model | Protocol Suite |
| Most Used | No | Yes |

# Result

The OSI model provides a theoretical framework for understanding network communication, whereas the TCP/IP model is the practical networking architecture used for Internet communication. TCP/IP combines some OSI layers, making it simpler and more efficient for real-world implementation.

# Conclusion

The experiment successfully compared the OSI and TCP/IP reference models based on layers, functions, protocols, and implementation. It was observed that while the OSI model is useful for learning and network design, the TCP/IP model is widely implemented in modern computer networks due to its simplicity, scalability, and support for Internet communication.

# Experiment No. 02

## Title

Demonstrate Different Types of Network Topologies and Types of Transmission Media Using Cisco Packet Tracer (Compulsory)

## Date of Completion

# Objectives

- To study different types of network topologies.

- To understand the characteristics of various transmission media.

- To create different network topologies using Cisco Packet Tracer.

- To observe communication between network devices.

- To analyze the advantages and disadvantages of each topology.

# Problem Statement

Design and demonstrate various network topologies and transmission media using Cisco Packet Tracer. Verify successful communication between connected devices and compare their characteristics.

# Software Requirements

- Cisco Packet Tracer 8.x or later

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

# Theory

## Introduction

A network topology refers to the physical or logical arrangement of devices in a computer network. The choice of topology affects network performance, scalability, reliability, and maintenance.

Transmission media is the communication path through which data travels from one device to another. It can be classified as guided (wired) or unguided (wireless).

# Types of Network Topologies

## 1. Bus Topology

### Description

All devices are connected to a single backbone cable.

### Advantages

- Easy to install

- Low cost

- Requires less cable

### Disadvantages

- Backbone failure affects the entire network

- Difficult fault detection

- Limited scalability

## 2. Star Topology

### Description

All devices are connected to a central hub or switch.

### Advantages

- Easy troubleshooting

- High performance

- Easy to expand

- Failure of one cable affects only one device

### Disadvantages

- Central device failure stops the network

- More cabling required

## 3. Ring Topology

### Description

Each device is connected to two neighboring devices, forming a ring.

### Advantages

- Orderly data transmission

- No collisions

### Disadvantages

- One node failure may disrupt communication

- Difficult maintenance

## 4. Mesh Topology

### Description

Every device is connected to every other device.

### Advantages

- Highly reliable

- Multiple communication paths

- Excellent fault tolerance

### Disadvantages

- Expensive

- Complex installation

- Large amount of cabling

## 5. Tree Topology

### Description

Hierarchical arrangement of star networks.

### Advantages

- Easy expansion

- Good scalability

- Suitable for large organizations

### Disadvantages

- Backbone dependency

- Complex maintenance

## 6. Hybrid Topology

### Description

Combination of two or more topologies.

### Advantages

- Flexible

- Scalable

- Reliable

### Disadvantages

- Costly

- Complex design

# Types of Transmission Media

Transmission media are classified into:

## A. Guided (Wired) Media

### 1. Twisted Pair Cable (UTP/STP)

- Used in LANs

- Low cost

- Supports Ethernet communication

- Maximum distance: 100 meters

Examples:

- Cat5e

- Cat6

- Cat6A

### 2. Coaxial Cable

- Better shielding than twisted pair

- Used in cable television and broadband

### 3. Optical Fiber Cable

- Very high speed

- Long-distance communication

- Immune to electromagnetic interference

Types:

- Single Mode Fiber

- Multi Mode Fiber

## B. Unguided (Wireless) Media

### 1. Radio Waves

Examples:

- Wi-Fi

- Bluetooth

### 2. Microwaves

Used for:

- Satellite Communication

- Cellular Networks

### 3. Infrared

Used for:

- TV Remote Controls

- Short-range communication

# Procedure

### Part A – Demonstrating Topologies

- Open Cisco Packet Tracer.

- Create a new project.

- Drag PCs, switches, and hubs onto the workspace.

- Design the following topologies:

  - Bus

  - Star

  - Ring

  - Mesh

  - Tree

- Connect devices using appropriate cables.

- Assign IP addresses to all PCs.

- Use the Add Simple PDU tool or Command Prompt (ping) to test connectivity.

- Observe successful packet transmission.

### Part B – Demonstrating Transmission Media

- Use different cable types available in Packet Tracer:

  - Copper Straight-Through

  - Copper Cross-Over

  - Fiber Optic

  - Console Cable

- Connect appropriate networking devices.

- Observe successful communication.

- Compare characteristics of different media.

# Algorithm

- Start Cisco Packet Tracer.

- Create network devices.

- Arrange devices according to the selected topology.

- Connect devices using suitable transmission media.

- Configure IP addresses.

- Verify connectivity using the Ping command or Simple PDU.

- Observe packet flow.

- Compare the performance of different topologies and transmission media.

- Record observations.

- End.

# Flowchart

Start
               |
               V
Open Cisco Packet Tracer
               |
               V
Create Network Topology
               |
               V
Connect Devices
               |
               V
Assign IP Addresses
               |
               V
Select Transmission Media
               |
               V
Test Connectivity
               |
               V
Observe Results
               |
               V
        Record Observations
               |
               V
             Stop

# Test Cases

| Test Case | Expected Result |
| --- | --- |
| Star topology ping | Successful communication |
| Ring topology ping | Successful communication |
| Mesh topology ping | Successful communication |
| Tree topology ping | Successful communication |
| Fiber optic connection | High-speed communication |
| Copper cable connection | LAN communication successful |

# Observation Table

## Network Topologies

| Topology | Devices Used | Connectivity | Result |
| --- | --- | --- | --- |
| Bus | PCs, Hub | Successful | Pass |
| Star | PCs, Switch | Successful | Pass |
| Ring | PCs | Successful | Pass |
| Mesh | PCs | Successful | Pass |
| Tree | Switches, PCs | Successful | Pass |

## Transmission Media

| Media | Type | Speed | Distance | Observation |
| --- | --- | --- | --- | --- |
| UTP Cable | Guided | High | 100 m | LAN Communication Successful |
| Fiber Optic | Guided | Very High | Several km | High-speed Communication |
| Coaxial Cable | Guided | Medium | Long | Reliable Communication |
| Wireless | Unguided | Varies | Depends on Signal | Mobility Supported |

# Applications

- Office LANs

- College Campus Networks

- Data Centers

- Internet Service Providers

- Smart Buildings

- Industrial Networks

- Wireless Networks

# Result

Different network topologies were successfully designed and tested using Cisco Packet Tracer. Various transmission media were demonstrated, and successful communication between devices was verified using the Ping command and Simple PDU tool.

# Conclusion

The experiment demonstrated the implementation of common network topologies and transmission media using Cisco Packet Tracer. It was observed that Star Topology is the most widely used in modern LANs due to its ease of maintenance and reliability, while Fiber Optic Cable provides the highest speed and longest transmission distance. Understanding these concepts is essential for designing efficient and scalable computer networks.

# Experiment No. 3

## Title

Setup a WAN Containing Wired and Wireless LANs Using Cisco Packet Tracer and Demonstrate Packet Transfer from LAN1 (Wired LAN) to LAN2 (Wireless LAN)

## Date of Completion

# Objectives

- To understand the concept of Wide Area Network (WAN).

- To configure a WAN connecting two geographically separated LANs.

- To configure both wired and wireless LANs using Cisco Packet Tracer.

- To assign IP addresses and configure routers for communication.

- To demonstrate successful packet transfer from a wired LAN to a wireless LAN.

# Problem Statement

Design and configure a WAN consisting of one Wired LAN (LAN1) and one Wireless LAN (LAN2) using Cisco Packet Tracer. Configure all required networking devices and verify packet transfer between the two LANs.

# Software Requirements

- Cisco Packet Tracer 8.x or later

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

# Theory

## Introduction

A Wide Area Network (WAN) connects multiple Local Area Networks (LANs) over large geographical distances using routers and communication links. WANs enable communication between different offices, campuses, or cities.

In this experiment:

- LAN1 is a wired network using a switch.

- LAN2 is a wireless network using a wireless router/access point.

- Two routers are connected through a WAN link.

- Data packets are transmitted from a wired PC in LAN1 to a wireless laptop in LAN2.

# Network Diagram (Logical)

WAN Link
        --------------------------
        |                        |
    Router0 ---------------- Router1
       |                          |
    Switch                    Wireless Router
   /      \                  /      |      \
 PC0      PC1           Laptop0  Laptop1  Smartphone

# IP Addressing Scheme

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Router0 | G0/0 | 192.168.1.1 | 255.255.255.0 |
| Router0 | S0/0/0 | 10.0.0.1 | 255.255.255.252 |
| Router1 | S0/0/0 | 10.0.0.2 | 255.255.255.252 |
| Router1 | G0/0 | 192.168.2.1 | 255.255.255.0 |
| PC0 | NIC | 192.168.1.10 | 255.255.255.0 |
| PC1 | NIC | 192.168.1.11 | 255.255.255.0 |
| Laptop0 | Wireless | 192.168.2.10 | 255.255.255.0 |
| Laptop1 | Wireless | 192.168.2.11 | 255.255.255.0 |

Default Gateway:

- LAN1 → 192.168.1.1

- LAN2 → 192.168.2.1

# Procedure

### Step 1: Create the Network

- Open Cisco Packet Tracer.

- Add:

  - 2 Routers

  - 1 Switch

  - 1 Wireless Router (or Access Point)

  - 2 PCs

  - 2 Laptops

### Step 2: Connect Devices

- PC0 → Switch

- PC1 → Switch

- Switch → Router0 (Copper Straight-Through)

- Router0 ↔ Router1 (Serial DCE/DTE Cable)

- Router1 → Wireless Router

- Connect laptops to the wireless network using the configured SSID.

### Step 3: Configure Router Interfaces

Assign IP addresses to:

- Router0 LAN Interface

- Router0 Serial Interface

- Router1 Serial Interface

- Router1 LAN Interface

Enable all interfaces using:

no shutdown

### Step 4: Configure Routing

Configure static routes.

Router0

ip route 192.168.2.0 255.255.255.0 10.0.0.2

Router1

ip route 192.168.1.0 255.255.255.0 10.0.0.1

### Step 5: Configure Wireless LAN

- Configure SSID (e.g., SPPU_WIFI).

- Set WPA2 password (e.g., password123).

- Connect laptops to the wireless network.

- Verify successful association.

### Step 6: Assign IP Addresses

Configure IP addresses manually or using DHCP for all end devices.

### Step 7: Test Connectivity

- Ping Router0 from PC0.

- Ping Router1 from Laptop0.

- Ping Laptop0 from PC0.

- Use Add Simple PDU to observe packet transfer from LAN1 to LAN2.

# Algorithm

- Open Cisco Packet Tracer.

- Place routers, switch, wireless router, PCs, and laptops.

- Connect devices using appropriate cables.

- Configure IP addresses.

- Configure router interfaces.

- Enable interfaces.

- Configure static routing.

- Configure wireless network settings.

- Connect wireless devices.

- Test communication using ping and Simple PDU.

- Observe packet transfer.

- Record the results.

# Flowchart

Start
             |
             V
Open Cisco Packet Tracer
             |
             V
Create Wired and Wireless LANs
             |
             V
Connect Routers Using WAN Link
             |
             V
Configure IP Addresses
             |
             V
Configure Routing
             |
             V
Configure Wireless Network
             |
             V
Connect Wireless Devices
             |
             V
Test Connectivity (Ping/PDU)
             |
             V
Packet Successfully Transferred
             |
             V
          Stop

# Test Cases

| Test Case | Expected Result |
| --- | --- |
| PC0 ping Router0 | Successful |
| Laptop0 ping Router1 | Successful |
| PC0 ping Laptop0 | Successful |
| PC1 ping Laptop1 | Successful |
| Simple PDU from LAN1 to LAN2 | Successful Delivery |

# Observation Table

| Source Device | Destination Device | Protocol | Result |
| --- | --- | --- | --- |
| PC0 | Router0 | ICMP | Pass |
| Router0 | Router1 | Serial | Pass |
| Router1 | Laptop0 | ICMP | Pass |
| PC0 | Laptop0 | Ping | Pass |
| PC1 | Laptop1 | Ping | Pass |

# Applications

- Corporate branch office connectivity

- University campus networks

- Banking networks

- Government organizations

- Internet Service Provider (ISP) networks

- Smart office communication

# Result

A WAN connecting a Wired LAN (LAN1) and a Wireless LAN (LAN2) was successfully configured using Cisco Packet Tracer. Static routing enabled communication between both LANs, and packet transfer from the wired network to the wireless network was verified using the Ping command and Simple PDU tool.

# Conclusion

The experiment demonstrated how routers interconnect separate LANs through a WAN. Both wired and wireless networks were successfully configured, and reliable communication between devices on different LANs was achieved. This experiment provides practical understanding of WAN architecture, routing, IP addressing, and integration of wired and wireless networks.

# Experiment No. 4

## Title

Capture and Analyze Ethernet Frames to Study Physical Addressing Using Wireshark Packet Analyzer Tool

## Date of Completion

# Objectives

- To understand the structure of an Ethernet frame.

- To study physical (MAC) addressing in Ethernet communication.

- To capture live network packets using Wireshark.

- To analyze source and destination MAC addresses.

- To observe Ethernet frame fields and their functions.

# Problem Statement

Capture Ethernet frames using the Wireshark Packet Analyzer tool and analyze the frame structure, including source and destination MAC addresses, EtherType, and payload to understand physical addressing in a LAN.

# Software Requirements

- Wireshark Packet Analyzer

- Windows/Linux Operating System

- Web Browser or Command Prompt

# Hardware Requirements

- Personal Computer/Laptop

- Network Interface Card (NIC)

- Wired or Wireless Internet Connection

# Theory

## Introduction

An Ethernet frame is the basic unit of data transmission in a Local Area Network (LAN). It operates at the Data Link Layer (Layer 2) of the OSI model and uses MAC (Media Access Control) addresses for communication between devices on the same network.

Wireshark is a network protocol analyzer that captures and displays network packets in real time, allowing users to inspect Ethernet frames and understand network communication.

# Ethernet Frame Format

| Field | Size | Description |
| --- | --- | --- |
| Preamble | 7 Bytes | Synchronizes communication between sender and receiver |
| Start Frame Delimiter (SFD) | 1 Byte | Indicates the start of the frame |
| Destination MAC Address | 6 Bytes | Physical address of the receiver |
| Source MAC Address | 6 Bytes | Physical address of the sender |
| EtherType/Length | 2 Bytes | Identifies the upper-layer protocol (e.g., IPv4, ARP) |
| Data (Payload) | 46–1500 Bytes | Actual data being transmitted |
| Frame Check Sequence (FCS) | 4 Bytes | Error detection using CRC |

# Physical (MAC) Address

A MAC (Media Access Control) Address is a unique 48-bit (6-byte) hexadecimal address assigned to a network interface card (NIC).

Example:

- Source MAC: 00:1A:2B:3C:4D:5E

- Destination MAC: F8:32:E4:7A:9B:1C

Characteristics:

- Globally unique

- Used for communication within a LAN

- Operates at the Data Link Layer (Layer 2)

# Procedure

- Install and open Wireshark.

- Select the active network interface (Ethernet or Wi-Fi).

- Click Start Capturing Packets.

- Generate network traffic by:

  - Opening a website, or

  - Running the ping command (e.g., ping google.com).

- Stop packet capture after a few seconds.

- Apply the filter:

eth

- Select an Ethernet frame from the captured packets.

- Expand the Ethernet II section.

- Observe:

  - Destination MAC Address

  - Source MAC Address

  - EtherType

- Record the observations.

# Algorithm

- Start Wireshark.

- Select the active network interface.

- Begin packet capture.

- Generate network traffic.

- Stop packet capture.

- Filter Ethernet packets.

- Analyze Ethernet frame fields.

- Record MAC addresses and protocol details.

- Draw conclusions.

- End.

# Flowchart

Start
             |
             V
      Open Wireshark
             |
             V
 Select Network Interface
             |
             V
    Start Packet Capture
             |
             V
 Generate Network Traffic
             |
             V
     Stop Packet Capture
             |
             V
   Apply Ethernet Filter
             |
             V
 Analyze Ethernet Frame
             |
             V
 Record Observations
             |
             V
            Stop

# Observation Table

| Parameter | Observation |
| --- | --- |
| Source MAC Address | XX:XX:XX:XX:XX:XX |
| Destination MAC Address | XX:XX:XX:XX:XX:XX |
| EtherType | IPv4 (0x0800) / ARP (0x0806) |
| Frame Length | ______ Bytes |
| Payload Size | ______ Bytes |
| Protocol | Ethernet II |

Note: Replace the MAC addresses and frame details with the values observed during packet capture.

# Test Cases

| Test Case | Expected Result |
| --- | --- |
| Start packet capture | Packets are captured successfully |
| Open a website / Ping a host | Ethernet frames are generated |
| Apply eth filter | Only Ethernet frames are displayed |
| View Ethernet II details | Source and Destination MAC addresses are visible |
| Analyze EtherType | Correct upper-layer protocol is identified |

# Sample Output

Ethernet II

Destination: 34:17:eb:12:ab:56
Source:      d8:5d:e2:78:45:11
Type: IPv4 (0x0800)

Internet Protocol Version 4
Transmission Control Protocol
Hypertext Transfer Protocol

# Applications

- Network troubleshooting

- MAC address identification

- LAN traffic analysis

- Protocol analysis

- Network security monitoring

- Performance analysis

# Result

Ethernet frames were successfully captured using the Wireshark Packet Analyzer. The source and destination MAC addresses, EtherType, and other frame fields were analyzed, demonstrating how physical addressing is used for communication within a Local Area Network.

# Conclusion

The experiment successfully demonstrated the capture and analysis of Ethernet frames using Wireshark. It provided practical understanding of Ethernet frame structure, MAC addressing, and the role of the Data Link Layer in LAN communication. Wireshark proved to be an effective tool for monitoring and analyzing network traffic.

# Experiment No. 5

## Title

Write a Program for Error Detection and Correction for 7/8-bit ASCII Codes Using Hamming Code (Compulsory)

## Date of Completion

# Objectives

- To understand the concept of error detection and correction.

- To study the Hamming Code technique.

- To implement Hamming Code for 7/8-bit ASCII data.

- To detect and correct a single-bit error.

- To verify the correctness of transmitted data after error correction.

# Problem Statement

Write a program to perform error detection and correction for 7/8-bit ASCII codes using the Hamming Code technique. The program should generate Hamming code, detect a single-bit error during transmission, and correct the erroneous bit.

# Software Requirements

- GCC Compiler / Code::Blocks / Visual Studio Code

- C Compiler

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

# Theory

## Introduction

During data transmission, bits may be altered due to noise or interference. Error Detection and Correction techniques help ensure reliable communication by identifying and correcting transmission errors.

Hamming Code, developed by Richard W. Hamming, is a forward error correction technique capable of detecting and correcting a single-bit error.

# Hamming Code

Hamming Code inserts parity bits at positions that are powers of two:

- P1 → Position 1

- P2 → Position 2

- P4 → Position 4

- P8 → Position 8

- ...

The parity bits are calculated using even parity (or odd parity as required).

# Working Principle

- Read the input data bits.

- Insert parity bits at positions 1, 2, 4, 8, ...

- Calculate parity values.

- Generate the Hamming code.

- Simulate transmission.

- Introduce a single-bit error (optional).

- Recalculate parity at the receiver.

- Locate the error position.

- Correct the erroneous bit.

- Display corrected data.

# Algorithm

- Read the input binary data (7/8-bit ASCII).

- Determine the required number of parity bits.

- Insert parity bits at positions 1, 2, 4, 8, ...

- Calculate parity values.

- Generate the Hamming code.

- Accept the received code.

- Recalculate parity bits.

- Compute the syndrome.

- If syndrome = 0, no error exists.

- Otherwise, identify the error position.

- Correct the erroneous bit.

- Display the corrected code.

# Flowchart

Start
             |
             V
     Read Input Data
             |
             V
 Insert Parity Bit Positions
             |
             V
 Calculate Parity Bits
             |
             V
 Generate Hamming Code
             |
             V
 Receive Data
             |
             V
 Check Parity
             |
             V
 Error Found?
      /            \
    No              Yes
    |                |
 Display Data   Correct Error
    |                |
     \              /
        Display Result
             |
             V
            Stop

# Program (C)

#include <stdio.h>
#include <math.h>

int main()
{
    int data[20], hamming[30];
    int m, r = 0, i, j, k = 0;

    printf("Enter number of data bits: ");
    scanf("%d", &m);

    while (pow(2, r) < (m + r + 1))
        r++;

    printf("Enter data bits:\n");
    for (i = m - 1; i >= 0; i--)
        scanf("%d", &data[i]);

    int total = m + r;

    k = 0;
    for (i = 1; i <= total; i++)
    {
        if ((i & (i - 1)) == 0)
            hamming[i] = 0;
        else
            hamming[i] = data[k++];
    }

    for (i = 0; i < r; i++)
    {
        int pos = pow(2, i);
        int parity = 0;

        for (j = 1; j <= total; j++)
        {
            if (j & pos)
                parity ^= hamming[j];
        }

        hamming[pos] = parity;
    }

    printf("\nGenerated Hamming Code:\n");

    for (i = total; i >= 1; i--)
        printf("%d", hamming[i]);

    printf("\n");

    int received[30];

    printf("\nEnter received code:\n");

    for (i = total; i >= 1; i--)
        scanf("%d", &received[i]);

    int error = 0;

    for (i = 0; i < r; i++)
    {
        int pos = pow(2, i);
        int parity = 0;

        for (j = 1; j <= total; j++)
        {
            if (j & pos)
                parity ^= received[j];
        }

        if (parity)
            error += pos;
    }

    if (error == 0)
    {
        printf("\nNo Error Detected.\n");
    }
    else
    {
        printf("\nError Detected at Position %d\n", error);

        received[error] ^= 1;

        printf("Corrected Code:\n");

        for (i = total; i >= 1; i--)
            printf("%d", received[i]);

        printf("\n");
    }

    return 0;
}

# Sample Input

Enter number of data bits:
4

Enter data bits:
1 0 1 1

Received Code:
1010101

# Sample Output

Generated Hamming Code:
1010101

Error Detected at Position 3

Corrected Code:
1011101

# Test Cases

| Test Case | Input | Expected Result |
| --- | --- | --- |
| No Error | Valid Hamming Code | No Error Detected |
| Single Bit Error | One bit changed | Error detected and corrected |
| Error at Parity Bit | Change parity bit | Error corrected |
| Error at Data Bit | Change data bit | Corrected successfully |

# Applications

- Computer Networks

- Memory Error Correction (ECC RAM)

- Satellite Communication

- Wireless Communication

- Data Storage Devices

- Digital Communication Systems

# Result

The program successfully generated the Hamming Code for the given data, detected a single-bit transmission error, identified its position, and corrected it.

# Conclusion

The experiment demonstrated the implementation of Hamming Code for error detection and correction. It verified that Hamming Code can efficiently detect and correct a single-bit error, improving the reliability of data transmission in computer networks and digital communication systems.

# Experiment No. 6

## Title

Write a Program to Simulate Go-Back-N and Selective Repeat Modes of Sliding Window Protocol in Peer-to-Peer Mode

## Date of Completion

# Objectives

- To understand the concept of the Sliding Window Protocol.

- To study the working of Go-Back-N (GBN) and Selective Repeat (SR) ARQ protocols.

- To simulate frame transmission between two peer nodes.

- To observe retransmission behavior in case of frame loss.

- To compare the performance of Go-Back-N and Selective Repeat protocols.

# Problem Statement

Write a program to simulate Go-Back-N and Selective Repeat modes of the Sliding Window Protocol in Peer-to-Peer mode. The program should demonstrate frame transmission, acknowledgments, and retransmission of lost frames.

# Software Requirements

- GCC Compiler / Code::Blocks / Visual Studio Code

- C Compiler

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

# Theory

## Introduction

The Sliding Window Protocol is a flow control protocol used at the Data Link Layer to ensure reliable data transmission. It allows multiple frames to be sent before receiving acknowledgments, improving network efficiency.

There are two common sliding window protocols:

- Go-Back-N (GBN) ARQ

- Selective Repeat (SR) ARQ

# Go-Back-N (GBN) ARQ

In Go-Back-N:

- The sender can transmit multiple frames without waiting for acknowledgments.

- If one frame is lost or corrupted, all subsequent frames are retransmitted starting from the lost frame.

- The receiver accepts only frames in sequence.

### Advantages

- Simple implementation

- Less receiver complexity

### Disadvantages

- Retransmits many correctly received frames

- Lower bandwidth efficiency

# Selective Repeat (SR) ARQ

In Selective Repeat:

- Only the lost or corrupted frame is retransmitted.

- The receiver stores correctly received out-of-order frames.

- Each frame has an individual acknowledgment.

### Advantages

- Efficient bandwidth utilization

- Fewer retransmissions

### Disadvantages

- More complex implementation

- Requires larger receiver buffer

# Algorithm

### Go-Back-N

- Read the number of frames.

- Read the window size.

- Transmit frames within the window.

- If acknowledgment is received, slide the window.

- If a frame is lost, retransmit that frame and all subsequent frames.

- Repeat until all frames are acknowledged.

### Selective Repeat

- Read the number of frames.

- Read the window size.

- Send all frames in the window.

- Receive acknowledgments individually.

- Retransmit only the lost frame.

- Continue until all frames are acknowledged.

# Flowchart

Start
               |
               V
      Read Number of Frames
               |
               V
      Select Protocol (GBN/SR)
               |
               V
       Send Frames in Window
               |
               V
     Receive Acknowledgment
               |
               V
        Error Detected?
          /          \
        No            Yes
        |              |
 Slide Window    Retransmit Frame(s)
        |              |
         \            /
          Continue Transmission
               |
               V
      All Frames Delivered?
          /           \
        No             Yes
        |               |
     Continue          Stop

# Program (C)

#include <stdio.h>

void goBackN(int frames, int window)
{
    int i = 1;

    printf("\n--- Go-Back-N Simulation ---\n");

    while(i <= frames)
    {
        printf("\nSending Frames: ");

        int j;
        for(j = i; j < i + window && j <= frames; j++)
            printf("%d ", j);

        int lost;
        printf("\nEnter Lost Frame (0 if none): ");
        scanf("%d", &lost);

        if(lost == 0)
        {
            printf("Acknowledgment Received.\n");
            i += window;
        }
        else
        {
            printf("Frame %d Lost.\n", lost);
            printf("Retransmitting Frames %d to %d\n", lost, (i + window - 1 > frames) ? frames : i + window - 1);
            i = lost;
        }
    }

    printf("\nAll Frames Transmitted Successfully.\n");
}

void selectiveRepeat(int frames, int window)
{
    int ack[100] = {0};
    int count = 0;

    printf("\n--- Selective Repeat Simulation ---\n");

    while(count < frames)
    {
        int i;

        for(i = 1; i <= frames; i++)
        {
            if(ack[i] == 0)
            {
                printf("\nSending Frame %d\n", i);

                int received;
                printf("Received Successfully? (1=Yes,0=No): ");
                scanf("%d", &received);

                if(received)
                {
                    ack[i] = 1;
                    count++;
                    printf("ACK %d Received\n", i);
                }
                else
                {
                    printf("Frame %d will be retransmitted.\n", i);
                }
            }
        }
    }

    printf("\nAll Frames Successfully Received.\n");
}

int main()
{
    int frames, window, choice;

    printf("Enter Number of Frames: ");
    scanf("%d", &frames);

    printf("Enter Window Size: ");
    scanf("%d", &window);

    printf("\n1. Go-Back-N");
    printf("\n2. Selective Repeat");
    printf("\nEnter Choice: ");
    scanf("%d", &choice);

    switch(choice)
    {
        case 1:
            goBackN(frames, window);
            break;

        case 2:
            selectiveRepeat(frames, window);
            break;

        default:
            printf("Invalid Choice");
    }

    return 0;
}

# Sample Input

Enter Number of Frames: 5
Enter Window Size: 3

1. Go-Back-N
2. Selective Repeat

Enter Choice: 1

Lost Frame: 2

# Sample Output (Go-Back-N)

--- Go-Back-N Simulation ---

Sending Frames: 1 2 3

Frame 2 Lost.
Retransmitting Frames 2 to 3

Sending Frames: 2 3 4

Acknowledgment Received.

Sending Frames: 5

Acknowledgment Received.

All Frames Transmitted Successfully.

# Sample Output (Selective Repeat)

--- Selective Repeat Simulation ---

Sending Frame 1
ACK 1 Received

Sending Frame 2
Frame 2 will be retransmitted.

Sending Frame 3
ACK 3 Received

Retransmitting Frame 2
ACK 2 Received

All Frames Successfully Received.

# Test Cases

| Test Case | Expected Result |
| --- | --- |
| No frame loss | All frames transmitted successfully |
| Frame lost in Go-Back-N | Lost frame and all subsequent frames retransmitted |
| Frame lost in Selective Repeat | Only the lost frame retransmitted |
| Window size smaller than frames | Frames transmitted in multiple windows |

# Applications

- Reliable data transmission

- Data Link Layer protocols

- Transport Layer communication

- Wireless communication systems

- Satellite communication

- High-speed computer networks

# Result

The program successfully simulated both Go-Back-N and Selective Repeat sliding window protocols. It demonstrated frame transmission, acknowledgment handling, and retransmission according to the selected protocol.

# Conclusion

The experiment demonstrated the working of Go-Back-N and Selective Repeat ARQ protocols in a peer-to-peer environment. It was observed that Go-Back-N retransmits the lost frame along with all subsequent frames, whereas Selective Repeat retransmits only the lost frame, resulting in better bandwidth utilization and improved transmission efficiency.

# Experiment No. 7

## Title

Write a Program to Implement Link State / Distance Vector Routing Protocol to Find Suitable Path for Transmission

## Date of Completion

# Objectives

- To understand the concept of routing in computer networks.

- To study the working of Distance Vector Routing and Link State Routing protocols.

- To implement a routing algorithm to determine the shortest path.

- To calculate the minimum cost between network nodes.

- To analyze routing tables generated by the algorithm.

# Problem Statement

Write a program to implement a Link State Routing Protocol (Dijkstra's Algorithm) or Distance Vector Routing Protocol (Bellman-Ford Algorithm) to determine the shortest path between nodes in a network.

# Software Requirements

- GCC Compiler / Code::Blocks / Visual Studio Code

- C Compiler

- Windows/Linux Operating System

# Hardware Requirements

- Personal Computer/Laptop

- Minimum 4 GB RAM

# Theory

## Introduction

Routing is the process of selecting the best path for transmitting data packets from a source node to a destination node in a network.

Routing protocols help routers exchange information and build routing tables to determine the most efficient path.

There are two major classes of routing protocols:

- Distance Vector Routing

- Link State Routing

# Distance Vector Routing

Distance Vector Routing is based on the Bellman-Ford Algorithm.

### Characteristics

- Each router maintains a routing table.

- Routers periodically exchange routing information with neighboring routers.

- The routing table contains:

  - Destination

  - Distance (Cost)

  - Next Hop

### Example Protocol

- RIP (Routing Information Protocol)

### Advantages

- Simple implementation

- Low memory requirement

### Disadvantages

- Slow convergence

- Count-to-infinity problem

# Link State Routing

Link State Routing uses Dijkstra's Shortest Path Algorithm.

### Characteristics

- Every router maintains the complete network topology.

- Each router independently computes the shortest path.

- Faster convergence than Distance Vector.

### Example Protocol

- OSPF (Open Shortest Path First)

### Advantages

- Fast convergence

- Loop-free routing

- Better scalability

### Disadvantages

- Higher memory usage

- More complex implementation

# Algorithm (Dijkstra's Link State Routing)

- Read the number of nodes.

- Read the network cost matrix.

- Select the source node.

- Initialize distances.

- Mark the source node as visited.

- Select the nearest unvisited node.

- Update distances to neighboring nodes.

- Repeat until all nodes are visited.

- Display the shortest distance from the source to every destination.

# Flowchart

Start
              |
              V
    Read Number of Nodes
              |
              V
   Read Cost Adjacency Matrix
              |
              V
      Select Source Node
              |
              V
   Initialize Distance Table
              |
              V
 Select Minimum Cost Node
              |
              V
 Update Neighbor Distances
              |
              V
 All Nodes Visited?
        /          \
      No            Yes
      |              |
   Repeat      Display Shortest Path
                     |
                     V
                    Stop

# Program (C) – Link State Routing using Dijkstra's Algorithm

#include <stdio.h>

#define MAX 20
#define INF 9999

int main()
{
    int cost[MAX][MAX], distance[MAX], visited[MAX];
    int n, source;
    int i, j, count, min, next;

    printf("Enter number of nodes: ");
    scanf("%d", &n);

    printf("Enter Cost Matrix:\n");

    for(i = 0; i < n; i++)
    {
        for(j = 0; j < n; j++)
        {
            scanf("%d", &cost[i][j]);

            if(cost[i][j] == 0 && i != j)
                cost[i][j] = INF;
        }
    }

    printf("Enter Source Node (0-%d): ", n - 1);
    scanf("%d", &source);

    for(i = 0; i < n; i++)
    {
        distance[i] = cost[source][i];
        visited[i] = 0;
    }

    distance[source] = 0;
    visited[source] = 1;

    for(count = 1; count < n; count++)
    {
        min = INF;

        for(i = 0; i < n; i++)
        {
            if(distance[i] < min && !visited[i])
            {
                min = distance[i];
                next = i;
            }
        }

        visited[next] = 1;

        for(i = 0; i < n; i++)
        {
            if(!visited[i] && (min + cost[next][i] < distance[i]))
            {
                distance[i] = min + cost[next][i];
            }
        }
    }

    printf("\nShortest Distance from Node %d\n", source);

    for(i = 0; i < n; i++)
    {
        printf("To Node %d = %d\n", i, distance[i]);
    }

    return 0;
}

# Sample Input

Enter number of nodes:
4

Enter Cost Matrix:

0 2 5 1
2 0 3 2
5 3 0 3
1 2 3 0

Enter Source Node:
0

# Sample Output

Shortest Distance from Node 0

To Node 0 = 0
To Node 1 = 2
To Node 2 = 4
To Node 3 = 1

# Test Cases

| Test Case | Input | Expected Result |
| --- | --- | --- |
| Small Network | 4 Nodes | Shortest path calculated |
| Large Network | 8 Nodes | Routing table generated |
| Disconnected Node | Infinite Cost | No reachable path |
| Fully Connected Network | Cost Matrix | Minimum distances displayed |

# Observation Table

| Source Node | Destination Node | Shortest Distance |
| --- | --- | --- |
| 0 | 1 | 2 |
| 0 | 2 | 4 |
| 0 | 3 | 1 |

# Applications

- Internet routing

- OSPF routing protocol

- Enterprise networks

- Data center networks

- ISP backbone routing

- Wireless mesh networks

# Result

The program successfully implemented the Link State Routing Protocol using Dijkstra's Algorithm. It calculated the shortest path from the selected source node to all other nodes and displayed the corresponding minimum distances.

# Conclusion

The experiment demonstrated the implementation of a Link State Routing Protocol for finding the shortest transmission path. It showed how Dijkstra's Algorithm efficiently computes optimal routes based on link costs. Compared to Distance Vector Routing, Link State Routing provides faster convergence and more accurate routing information, making it suitable for large-scale networks.